PRACTICAL - 4

AIM: Demonstrate the Dynamic routing configuration using RIP and OSPF protocol using cisco packet tracer

THEORY:

DYNAMIC ROUTING

- It is a networking technique that provides optimal data routing. Unlike static routing, dynamic routing enables routers to select paths according to real-time logical network layout changes.

- Dynamic routing uses multiple algorithms and protocols. The most popular are Routing Information Protocol (RIP) and Open Shortest Path First (OSPF).

- Dynamic routing protocols allow routers to share information about the network with other routers to allow them to select the best path to reach a destination.

RIP:

- Routing Information Protocol version 2 (RIPv2) is an old routing protocol.

- RIPv2 suffers from scalability issues due to a relatively low maximum hop count of 15 routing devices.

OSPF:

- OSPF is the dynamic routing protocol used in large to very large IP networks.

- The protocol uses a link-state database and link-state advertisements to map the network topology.

Advantages of Dynamic Routing:

- Allows the exchange of routing information whenever the network experiences a change in topology.

- Since the routes do not have to be configured manually, there is less administrative overhead.

- Less error-prone than static routing.

- Allows scalability since there is less administrative overhead involved.

CONFIGURATION USING RIPv2

TOPOLOGY:

STEPS OF CONFIGURATION

- First of all to create a topology, click on the device and drop on workplace and connect all the devices with the necessary cables.

- Then configure all the router using CLI and give the ip address as mention in the topology diagram.

- Router 1: 	fa0/0 10.0.0.1

fa0/1 20.0.0.1

- Router 2: 	fa0/0 20.0.0.2

fa0/1 30.0.0.1

- Provide ip address and default gateway to the laptops.

- Laptop 1:     ip address is 10.0.0.2

default gateway is 10.0.0.1

- Laptop 2:     ip address is 30.0.0.2

default gateway is 30.0.0.1

- Configuration of RIPv2 on router 0 and router 1 are respectively,

- Verify RIP configuration.

CHECK NETWORK TOPOLOGY

We can check if the connection is working properly by two ways:

- MESSAGE PASSING

To check the connections are working properly or not drop one package on a Laptop 0 and receive it from the Laptop 1.

- USING PING COMMAND

Write a command ping 10.0.0.2 (ip address of the destination Laptop) from the command prompt of Laptop having ip address 30.0.0.2 (the source Laptop).

CONFIGURATION USING RIPv2

TOPOLOGY:

STEPS OF CONFIGURATION

- Configuration of OSPF on router 0 and router 1 are respectively,

- Verify OSPF configuration.

CHECK NETWORK TOPOLOGY

- MESSAGE PASSING

To check the connections are working  properly or not drop one package on a Laptop 1 and receive it from the Laptop 0.

- USING PING COMMAND

Write a command ping 30.0.0.2 (ip address of the destination Laptop) from the command prompt of Laptop having ip address 10.0.0.2 (the source Laptop).

CONCLUSION:

Through this practical I learned about how to configure dynamic routing of routers using RIPv2 and OSPF protocol in different networks.