PRACTICAL - 6

AIM: Demonstrate the static and dynamic configuration of NAT using cisco packet tracer

THEORY:

NAT

What is Network Address Translation (NAT)?

- NAT (Network Address Translation) is a process of changing the source and destination IP addresses and ports.

- Address translation reduces the need for IPv4 public addresses and hides private network address ranges.

- The process is usually done by routers or firewalls.

How does it work?

- It is suitable only for small network.

- If a link fails it cannot reroute the traffic.

There are three types of address translation.

- Static NAT – translates one private IP address to a public one. The public IP address is always the same.

- Dynamic NAT – private IP addresses are mapped to the pool of public IP addresses.

- Port Address Translation (PAT)– one public IP address is used for all internal devices, but a different port is assigned to each private IP address. Also known as NAT Overload.

STATIC CONFIGURATION OF NAT

TOPOLOGY:

STEPS OF CONFIGURATION

Provide IP address to all the Routers

- First of all to create a topology, click on the device and drop on workplace and connect all the devices with the necessary cables.

- Then configure router using CLI and give the ip address as mention in the topology diagram.

- Router: 	fa0/0 192.168.1.1

fa0/1 200.0.0.1

- Provide ip address and default gateway to the laptop.

- Laptop: 	ip address is 192.168.1.10

default gateway is 200.0.0.1

- Provide ip address and default gateway to the server.

- Server: 	ip address is 200.0.0.10

default gateway is 200.0.0.1

- Configuration of Static NAT on router 0,

- Verify Static NAT configuration.

CHECK NETWORK TOPOLOG

We can check if the connection is working properly by two ways:

- MESSAGE PASSING

To check the connections are working  properly or not drop one package on a Laptop and receive it from the Server.

- CHECKING NETWORK TOPOLOGY USING PING COMMAND

Write a command ping 200.0.0.10 (ip address of the Server) from the command prompt of Laptop having ip address 192.168.1.10 (the source Laptop).

DYNAMIC CONFIGURATION OF NAT

TOPOLOGY:

STEPS OF CONFIGURATION

Provide IP address to all the Routers

- First of all to create a topology, click on the device and drop on workplace and connect all the devices with the necessary cables.

- Then configure router using CLI and give the ip address as mention in the topology diagram.

- Router: 	fa0/0 192.168.1.1

fa0/1 200.0.0.1

- Provide ip address and default gateway to the laptop.

- Laptop 1: 	ip address is 192.168.1.2

default gateway is 200.0.0.1

- Laptop 2: 	ip address is 192.168.1.3

default gateway is 200.0.0.1

- Provide ip address and default gateway to the server.

- Server: 	ip address is 200.0.0.10

default gateway is 200.0.0.1

- Configuration of Static NAT on router 0,

- Verify Static NAT configuration.

CHECK NETWORK TOPOLOG

We can check if the connection is working properly by two ways:

- MESSAGE PASSING

To check the connections are working  properly or not drop the packets from laptop to server.

- CHECKING NETWORK TOPOLOGY USING PING COMMAND

Write a command ping 200.0.0.10 (ip address of the Server) from the command prompt of Laptop having ip address 192.168.1.2 (the source Laptop 0).

Just when ping run <debug ip nat> command on router:

CONCLUSION:

Through this practical I learned about how to configure Dynamic and Static NAT (Network Address Translation) in cisco packet tracer.